# DNA EXTRACTION FROM LYMPHOCYTES

Cells should be washed and treated under the same conditions as for whole blood (see MANUAL DNA EXTRACTION FROM BLOOD OR LYMPHOCYTES with PHENOL/CHLOROFORM protocol).

### Equipment and Materials

- Saline solution:

- NaCl 3M   		50 ml

- Distilled water to 1000 ml

- SLR :

- TRIS 2M pH 7.6	10 ml

- MgCl2 1M		10 ml

- NaCl 3M		6.6 ml

- Distilled water to	2000 ml

- 50 ml SARSTEDT tubes

- Centrifuge

### Procedure

The cell suspension must be treated on the day it arrives, imperatively.

- Homogenize the culture flask with gentle manual agitation

- Transfer the culture to a 50 ml tube

- Centrifuge at 2000 rpm for 10 min

- Decant the supernatant

- Wash the pellet with 10 ml saline solution

- Centrifuge for 5 min at 2000 rpm

- Decant the supernatant

- Wash the pellet with 10 ml SLR

- Centrifuge for 5 min at 2000 rpm

- Decant the supernatant

- Cells may be frozen at –80°C at this stage, or be lysed for DNA extraction

DNA will be extracted from the cells using the same procedures as for cells from whole blood (see corresponding protocols: Salt Method, Phenol/Chloroform, Autogen)