# DNA EXTRACTION FROM MUSCLE

### Equipment and Materials

- Mortar

- Pestle

- 1.5 ml Eppendorf tubes

- Isopropanol

- Liquid nitrogen

- Phenol

- Proteinase K

- RSB buffer

- TE buffer

SOLUTIONS

RSB buffer

10 mM Tris-HCl, pH 7.4

10 mM NaCl

25 EDTA

10% SDS

(add 10mg/ml proteinase K right before use)

TE buffer

10 mM TRIS

1 mM EDTA

pH 8.5 with HCL

Keep solutions at room temperature.

### Procedure

- Crush approximately 50 mg (30-75 mg) frozen muscle with mortar and pestle to a fine powder. Keep muscle in liquid nitrogen and do not let it thaw. Keep mortar and pestle cold with liquid nitrogen.

- Add muscle to 0.2 RSB buffer containing 10 mg/ml proteinase K (use 1.5 ml Eppendorf tubes throughout the procedure).

- Incubate at 50°C for approximately 2 hours until solution is homogeneous.

- Add 25 µl 5M NaCl (0,5M final concentration)

- Extract with phenol: SEVAG (equal volumes, 250 µl phenol, 250 µl SEVAG).

- Mix by turning tubes upside down. Do not Vortex at any point.

- Centrifuge 10 minutes at max velocity. Remove aqueous phase – extract again with phenol: SEVAG, same as above.

- Remove aqueous phase and extract with ether (approximately 2 volumes).

- Add cold isopropanol – approximately 1:1 or 1:2 volumes (fill up the Eppendorf tube). Mix by turning tube upside down.

- After DNA precipitates, spool it out using micropipettes with flamed tips. [If not enough to spool out, centrifuge, remove supernatant and wash pellet with 70% Ethanol]. Immediately put into 70% Ethanol (approx. 0.5 ml).

- Centrifuge, dry pellet, resuspend in TE buffer (20-100 µl, depending on yield).